How to use a grinder

To use a disc sander, grinder or wire wheel proceed as follows:

- Grip the tool with both hands.

- Adopt a well-balanced stance and do not over-reach.

- Start the tool and allow it to reach full speed before bringing it in contact with the material.

- Bring the front section of the disc into contact with the work.

- Move the tool backward and forward along the length of the material, making each stroke a little further across the width of the required area.

- Overlap each stroke slightly.

- Keep the tool moving at all times.

- Do not let it rest in one position.

- Maintain an even light pressure.

- Lift the tool clear of the work before switching it off.